IOS SDK说明文档

SDK接入说明

只需将BleSDK这个文件夹导入到所需的项目内即可

相关函数说明

/*!

*  @method sharedManager:

*

*  @discussion Singleton pattern 单例模式

*

*/

+(BleSDK_J2301A *)sharedManager;

/*!

*  @method GetDeviceTime:

*

*  @discussion Get the time of the device  获取设备的时间

*

*/

GetDeviceTime;

/*!

*  @method SetDeviceTime:

*  @param deviceTime   time parameter  时间参数.

*  @discussion Set the time of the watch 设置设备的时间

*

*/

SetDeviceTime:(MyDeviceTime_J2301A)deviceTime;

/*!

*  @method GetPersonalInfo:

*

*  @discussion Get the personal information of the deive 获取设备的个人信息

*

*/

GetPersonalInfo;

/*!

*  @method SetPersonalInfo:

*  @param personalInfo   Personal basic information parameters   个人基本信息参数

*  @discussion Set up your device's personal information  设置设备的个人信息

*

*/

SetPersonalInfo:(MyPersonalInfo_J2301A)personalInfo;

/*!

*  @method GetStepGoal

*

*  @discussion Get your device's step goal 获取设备的步数目标

*

*/

GetStepGoal;

/*!

*  @method SetStepGoal:

*  @param stepGoal    target value of steps 步数的目标值.

*  @discussion Set a step goal  设置步数目标

*

*/

SetStepGoal:(int)stepGoal;

/*!

*  @method GetDeviceBatteryLevel

*

*  @discussion Get the battery of the watch  获取设备的电量

*

*/

GetDeviceBatteryLevel;

/*!

*  @method GetDeviceMacAddress

*

*  @discussion Get the Mac address of the watch  获取设备的Mac地址

*

*/

GetDeviceMacAddress;

/*!

*  @method GetDeviceVersion

*

*  @discussion  Get the version information of the watch 获取设备的版本信息

*

*/

GetDeviceVersion;

/*!

*  @method Reset

*

*  @discussion  Factory reset (factory reset will delete all data on the device) 恢复出厂设置(恢复出厂设置会让设备的数据全部删除)

*

*/

Reset;

/*!

*  @method MCUReset

*

*  @discussion MCU reset, restarting the device, will not delete the data stored in the watch    MCU复位，重启设备，不会删除设备存储的数据

*

*/

MCUReset;

/*!

*  @method GetAutomaticMonitoringWithDataType:

*  @param dataType   1 means heartRate  2 means spo2  3 means temperature  4 means HRV 1 表示心率 2 表示血氧饱和度 3 表示体温 4 表示 HRV

*  @discussion get  the automatic monitoring information set by the watch

*

*/

GetAutomaticMonitoringWithDataType:(int)dataType;

/*!

*  @method SetAutomaticMonitoring:

*  @param automaticMonitoring   Automatic measurement setting information of the watch  设备的自动测量设置信息

*  @discussion Set the automatic measurement setting information of the watch 设置设备的自动测量设置信息

*

*/

SetAutomaticHRMonitoring:(MyAutomaticMonitoring_J2301A)automaticMonitoring;

/*!

*  @method ppgWithMode::

*  @param ppgMode  1 表示开启ppg测量   2表示给设备发送测量结果  3表示停止ppg测量  4表示给设备发送ppg测量的进度  5表示退出ppg测量

*  @param ppgStatus 当 ppgMode=2或者 ppgMode=4的时候才有效。当ppgMode=2时，0表示测量失败  1 表示测量结果偏低  2表示测量结果正常 3表示测量结果偏高 。 当ppgMode=4时，ppgStatus表示测量的进度值，范围是0-100

*  @discussion Turn on ECG measurement 开启ECG测量

*

*/

ppgWithMode:(int)ppgMode  ppgStatus:(int)ppgStatus;

/*!

*  @method RealTimeDataWithType:

*  @param dataType   0: means off 1: when the number of steps changes, the watch will upload data (the data does not include the temperature value)  0:表示关闭  1:当步数发生变化时，设备会上传数据（数据不包含温度值）

*

*  @discussion Watch real-time data upload  实时数据上传

*

*/

RealTimeDataWithType:(int8_t)dataType;

/*!

*  @method manualMeasurementWithDataType::

*  @param dataType heartRateData menas heart rate  spo2Data menas spo2

*  @param measurementTime  The length of time to measure, the minimum value is 30 seconds

*  @param isOpen  When isOpen=YES, it means open measurement. When isOpen=NO, it means close the measurement.

*  @discussion Watch real-time data upload  实时数据上传

*

*/

manualMeasurementWithDataType:(MeasurementDataType_J2301A)dataType measurementTime:(int)measurementTime open:(BOOL)isOpen;

/*!

*  @method GetTotalActivityDataWithMode::

*  @param mode   0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50) 0x99: It means to delete all the total exercise data   0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有运动总数据

*  @param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get total exercise data    获取运动总数据

*

*/

GetTotalActivityDataWithMode:(int)mode withStartDate:(NSDate*)startDate;

/*!

*  @method GetDetailActivityDataWithMode::

*  @param mode    0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: Indicates to delete all detailed exercise data          0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有详细运动数据

*   @param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion  Get detailed exercise data   获取详细运动数据

*

*/

GetDetailActivityDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetDetailSleepDataWithMode::

*  @param mode    0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: Indicates to delete all sleep  data    0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有睡眠数据

*  @param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get sleep data  获取睡眠数据

*

*/

GetDetailSleepDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetContinuousHRDataWithMode::

*  @param mode   0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all continuous HR  data  0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有连续心率数据

*  @param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get continuous heart rate data  获取连续心率

*

*/

GetContinuousHRDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetSingleHRDataWithMode::

*  @param mode    0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all single HR  data  0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有单次心率数据

*  @param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get single heart rate data 获取单次心率数据

*

*/

GetSingleHRDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetHRVDataWithMode::

*  @param mode  0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all HRV data  0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有HRV数据

@param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this   parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get HRV monitoring data   获取HRV历史数据

*

*/

GetHRVDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetAutomaticSpo2DataWithMode::

*  @param mode  0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all  automatic Spo2 data   0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有自动血氧历史数据

@param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this   parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get automatic Spo2 historical data  获取自动Spo2历史数据

*

*/

GetAutomaticSpo2DataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetManualSpo2DataWithMode:

*  @param mode  0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all  manual Spo2 data   0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有手动血氧历史数据

@param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this   parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get manual Spo2 historical data  获取手动Spo2历史数据

*

*/

GetManualSpo2DataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetTemperatureDataWithMode::

*  @param mode    0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all  temperature data   0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有体温历史数据

@param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this   parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get temperature history data  获取体温的历史数据

*

*/

GetTemperatureDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method GetActivityModeDataWithMode::

*  @param mode  0: It means to start reading from the latest position (up to 50 sets of data) 2: It means to read next (when the total number of data is greater than 50)     0x99: means to delete all activity mode data  0:表示是从最新的位置开始读取(最多50组数据)  2:表示接着读取(当数据总数大于50的时候) 0x99:表示删除所有运动类型历史数据

@param  startDate  Read data from this date (note: this date must be exactly the same as the date of a piece of data stored in the watch, otherwise this   parameter will be invalid)    从这个日期开始读取数据(注意：这个日期一定是要与手环存储的某条数据的日期完全相同，否则这个参数将无效)

*  @discussion Get sports type historical data  获取运动类型历史数据

*

*/

GetActivityModeDataWithMode:(int)mode  withStartDate:(NSDate*)startDate;

/*!

*  @method startActivityMode:::

*  @param activityMode   sport mode

*  @param workMode  Working mode

*  @param activityTime  Set how long to exercise unit is minutes

*  @param breathParameter When workMode is 6, you need to set the parameters of breathing exercise  当workMode是6的时候需要设置呼吸锻炼的参数

*  @discussion Enter or exit multisport mode  进入或退出多运动模式

*

*/

startActivityMode:(ACTIVITYMODE_J2301A)activityMode WorkMode:(WORKMODE_J2301A)workMode  ActivityTime:(int)activityTime BreathParameter:(MyBreathParameter_J2301A)breathParameter ;

/*!

*  @method getActivityMode

*  @discussion Get current exercise mode  获取当前戒指的运动模式

*

*/

getActivityMode;

/*!

*  @method ClearAllHistoryData

*  @discussion  Clear all historical data  清除所有历史数据

*

*/

ClearAllHistoryData;

/*!

*  @method DataParsingWithData:

*  @param bleData Data returned by the watch  手环返回的数据

*  @discussion  data analysis 数据解析

*

*/

-(DeviceData_J2301A*)DataParsingWithData:(NSData*_Nullable)bleData;